[image: image1.jpg]COMMERCIAL ANTENSIFIER

MODEL CA-1B

INSTALLATION INSTRUCTIONS 2

1. The Model CA-18 amplifies all 12 channels using
cither 75 or 300 ohm line IN and OUT.

2. The Ca-18 provides line amplification at any point
in a Master TV System or in a deluxe home in-
stallation.

3. The input circuit handles up 10 40,000 microvolts
combined peak af 75 ohms or 80,000 microvols
at 300 ohms. Turning gain conirol counter-clock-
wise_permitshigher input with reduced gain.
CAUTION: Use small screwdriver for recessed slot
and do not force beyond stop,

4. The unit may be installed in B-T Model WH-1
(Weatherproof Housing) for outdoor use. To
facililate heat dissipation in poorly ventilated lo.
cations, the amplifir cover may be removed.

5. To maintain Hi and LO channel signal balance in

long_transmission lines, the B-T Model ME-1 or
ME-2 (Line Loss Equalizer) should be installed
before each CA-1B.

6. For eficient operation, the line voliage should be
nearly constant within the 110-120 range. If
voltage regulator is necessary, use a unit with low
harmonic content, such as Sola.

7. Connections for 75 and 300 ohm lines and strip
ping dimensions are shown below. For cable,
scrow in enclosed 11/U or 59/U grounding clamps
and attach ground lugs. Twist 59/U shield into
one pigtail, form into lug, crimp fightly, solder
and frim. For 11/U form ond solder o socond
pigtail on opposite side and also fin center con-
ductor. Make sure shield is soldered to clamp ot
top and bottom. Be careful not fo melt poly-
ethylene.

INPUT AND OUTPUT TERMINAL CONNECTIONS

TV TRANSMISSION LINES b toss per 100 1.

Nominat | 30 | 100 [ 200 | 500 [ 1000
we | me | me | me | e

(ones) |vme | vie | vwr | une [ uwe
kG 1170 75 |14 | 20 |30 | 51| 80
kG 5970 73 |27 |38 | 57 | 94 142 |
Ribbon' 300 | 8|2 |18 |32 50

Tubular 300 |7 [ 12|20 |31 46|
[Open wire’| 375 | 5[ s |12 | 20 30
3| 4

[Open wire’| 450 e e

11055 INCREASES 6% WHEN WET
HL0SS INCREASES 7 WHEN WET
2L05S INCREASES 1.5X WHEN WET




[image: image2.jpg]COMMERCIAL
ANTENSIFIER

MODEL CA-1B

SPECIFICATIONS

MODEL: CA-18 Commerial Antensfr
BANOWIDTH: Ch. 2.6 and Ch. 7-13
GAIN: 264 10" - 24db "N
NOISE FIGURE: b average.

GAIN CONTROL: 14db range

MAX. 75 OHM OUT. .7 V. ook .
MAX. 300 OHM OUT. 1.4 V. Feok.

MAX. 75 OHM IN. 40,000 microvols
MAX. 300 OHI IN: 80,000 micrvols
TUBES. Two 616 & Two 6BQ7A

w

n

1 9 o

POWER 117 ¥, 40 Crle AC. 40 ot
RUSE: 34 Amp, Exenal.
CABINET. oy Hammrione et
2k 87 x4 x5
weicH. 7 b
GUARANTEE. Standard 90 oy
PARTS LIST
PART SYMBOL DESCRIFTION
otar k1
et k2,416
Tioan 13
Tizon k4.5 17 b W 10%
Thos k75, Va1 i oK ok, oW, 105
e 11113 0K ohm, VoW, 20% P
1182041 512 82 o, 1AW, 10% =
iors) k14 100 ahm:
16153 w1 tooon i
st w1 bk s 3
it kot 230K ohm i
Liiotos 21 et 160 s 10 :
ey T30 o, | 3
S €1 fo 11,1, 13 Vs o H
Ihisos € 5 s et 10% g
Shideor 45,67 a0 8 1, 8% &
S1atan € 5 157404 57540 0w oy
2133104 €1, 17 530" el oY
Siiaoot A 19 T i 103 L
Sionor €2z V0 i, 10% £
S300008 Crow, 33b 166740 nia, TOOVD.C
aan0r €30 2w 10% :
i
Siiceos 11 &
S3io0rs 61 ]
4200001 71 Neon B, NE3 %
prER HECr 150 &
55V e i 173
4100008 V1, Vet aorx
4100008 V-3 va pymif e

BLONDER-TONGUE o

LABORATORIES, INC Al
NEWARKC 2. 1. 2 Rl

snet

2

€A

SA




